Scaleguard
Fitting and Operating Instructions

Please read thoroughly before installation

Components:

a) One Scaleguard unit with a 3 metre mains connection cable and plug.

b) One 3m length of signal cable.

c) Two screws and wall plugs to mount the unit on wall. A template is at the bottom of these instructions.

d) 4 cable ties (plus 2 spare ties) to secure signal cable to the pipe.

Installation Site

Locate the incoming main water pipe and choose a position for the coil (see notes below).  The unit can be installed up to 1 metre away from the pipe and requires a mains electrical outlet within 3 metres.  The coil can be installed on copper, plastic, steel or lead pipes.  If you have choices between plastic and copper pipe choose the plastic even if it is a bigger pipe as Scaleguard is most effective on plastic.  The pipe on which the coil is to be placed can be vertical, horizontal or diagonal and the box can be mounted vertically or horizontally.

[image: image1.jpg]



Important Notes Affecting Performance

1) For optimum results you require a minimum 10cm length of straight pipe clear of fittings, valves and if possible earth bonding clips.  Although the coil only requires 5cm of pipe you must have 2.5cm of clear pipe at each end.

2) Where possible keep the coil at least 15cm away from large iron and steel objects especially electrical motors (small pipes and sides of washing machines or cookers, etc are not a problem).

3) Any excess signal cable should be cut off and not left loose or coiled.

4) If you are unable to install the unit before the pipe work splits it is possible to treat two pipes by using a single coil on each pipe (see diagram on page 2)

Mounting the Unit

You can mount the unit on any convenient surface and it can be installed at any angle, above or below the coil but should be placed at least 15 cm away from the coil. 

Fixing Template

       +                                                                                        +
Installing the Coil

To fit the signal cable, strip one end of the signal cable to expose approximately 1cm of bare copper wire.  Twist this wire together, push back one of the terminal buttons and push the wire fully into either coloured terminal hole then release the button, polarity is not important.  Secure the signal cable to the pipe using one of the cable ties provided, then coil the signal cable tightly round the pipe with 10 or 11 close turns (more or less will reduce performance) and secure using a second cable tie.  Trim excess signal cable, bare the wire and insert cable into remaining terminal.

Mains Connection

If your Scaleguard is to be installed in a bathroom or a damp atmosphere, ensure that it is installed by a qualified electrician (the electrical circuitry within the unit is coated with a water proof resin, making it safe for use in such environments).  The installation of a new socket outlet or a fused spur if required should also be undertaken by a qualified electrician.  The Scaleguard can be plugged into a socket or if required remove the plug and wire into a fused spur.  Connect the unit to the mains supply and switch on.  The unit is designed to be switched on permanently.

Indicator Lights

The red power light indicates power is on and the green descaling signal light indicates the Scaleguard is functioning correctly and will only light up if the signal cable is connected.  If either of these lights fail, please check the fuse (3 amp) and all electrical connections. 

Installation Queries

In the event of any queries, please contact our help line on:

01491 419214

[image: image2.jpg]IHDOI |
- L =9 | =

Two Coil/Two '

- | Pipe installation





THE SCALEGUARD  GUARANTEES
PRODUCT GUARANTEE
The product guarantee is valid 90 days following the date of purchase. In the unlikely event that your Scaleguard becomes unserviceable (i.e. the red or green lights fails to illuminate) send the unit back for replacement, quoting the date of purchase. 
This guarantee does not extend to any other electrical or water appliance or device that may be in use by the user of Scaleguard and we accept no liability for the malfunction of any device other than Scaleguard which has, either directly or indirectly been sold to the user.

Scaleguard does not affect the portability or the natural mineral content of the water as there is nothing in contact with the water, therefore, we accept no responsibility for water quality.

The Scaleguard Performance Guide
Scaleguard treats all incoming water as it passes the coil and alters the characteristics of calcium so that it does not stick and form hard scale. It is important to remember that calcium is still in the water and will be visible in appliances that are not subject to the free flowing water, e.g. a kettle.

Your Scaleguard will:

1. Prevent any further build-up of hard scale.
2. Remove any existing scale within your water system.
3. Soften hard scale around taps, basins and toilets etc.
4. Make the water kinder to the skin and clothing.
How to check your Scaleguard is working for you:
The timescales vary according to water hardness and water consumption, but generally signs of descaling are seen within 4-6 weeks and often much earlier. Please not that Scalefguard may appear to become less effective after several weeks. This is because existing scale in the system is being removed so scale prevention is less effective during this period. Once the scale has been removed from the system full effectiveness will then return.
SCALEGUARD
INSTRUCTIONS & FITTING

GU

